Roof Plan:

- Lighting Protection:

Floor Plan(1st and 2nd):

Ground Floor Plan:

Dimensions:

1st and 2nd Floor:

Ground floor:

Fittings and Fixtures: Apartment Unit:

Garage Unit:

Roof corridor:

Apartment Corridor:

Conduit Layout:

Apartment Unit:

Normal conduit layout:

Emergency conduit layout:

Garage Unit:

Normal conduit layout:

Emergency conduit layout:

Roof corridor:

Apartment Unit Corridor:

1st and 2nd Floor:

Ground Floor:

Connection Diagrams:

- Normal switch Board Diagram:

Emergency Switch Board Diagrams:

Normal Sub-distribution Board Diagram:

Emergency Sub-distribution Board Diagram:

Normal Main Distribution Board Diagram:

Emergency Main Distribution Board Diagram:

LEGENDS for Fittings and Fixtures:

Legends for Conduits:

| C1=2x1.5 rm BYM +1.5 rm BYA ECC | ¾” |
| --- | --- |
| C2=4x1.5 rm BYM +1.5 rm BYA ECC | ¾” |
| C3=6x1.5 rm BYM +1.5 rm BYA ECC | ¾” |
| C5=2x6 rm BYM +6 rm BYA ECC | ¾” |
| C6=4x6 rm BYM +6 rm BYA ECC | 1” |
| C1,5= 2x1.5 rm BYM +1.5 rm BYA ECC & 2x6 rm BYM +6 rm BYA ECC | ¾” & ¾” |
| C1,6=2x1.5 rm BYM +1.5 rm BYA ECC & 4x6 rm BYM +6 rm BYA ECC | ¾” & 1” |
| C2,5=4x1.5 rm BYM +1.5 rm BYA ECC & 2x6 rm BYM +6 rm BYA ECC | ¾” & ¾” |
| C2,6=4x1.5 rm BYM +1.5 rm BYA ECC & 4x6 rm BYM +6 rm BYA ECC | ¾” & 1” |
| C7=2x16 rm BYM +16 rm BYA ECC | 2’’ |

N.B: The approximate and plausible dimension of the conduits are given above instead of the exact.

CALCULATIONS FOR LIGHT BULBS (LB) & FANS (F)

- Formulae:

Light Bulbs , E =  n*N*F*UF*LLFA   (Area  A in m^2)

Number of Fans , F = A100      (Area  A in sqft)

Illuminance  E

Light Loss Factor  LLF

Utilization Factor  UF

Number of lights per illuminaire  n

Flux  F

- BEDROOM- 1

Area : 16.5x10 ft^2 = 15.35 m^2

Illuminance , E = 100 Lumen/m^2

Light Loss Factor & Utilization Factor , LLF x UF = 0.7

Number of lights per illuminaire , n= 1

Flux = 1000 Lumen

Number of lights , N = 1.75

So, 2  Tube Lights are needed

Number of Fans =  1.65

So, 2 Fans are needed.

- BEDROOM- 2:

Area : 16.5x10 ft^2 = 15.35 m^2

Illuminance , E = 100 Lumen/m^2

Light Loss Factor & Utilization Factor , LLF x UF = 0.7

Number of lights per illuminaire, n= 1

Flux = 1250 Lumen

Number of lights , N = 1.75

So, 2  Tube Lights are needed

Number of Fans =  1.65

So, 2 Fans are needed.

- Guest Room:

Area : 16.5x10 ft^2 = 15.35 m^2

Illuminance , E = 100 Lumen/m^2

Light Loss Factor & Utilization Factor , LLF x UF = 0.7

Number of lights per illuminaire, n= 1

Flux = 1250 Lumen

Number of lights , N = 1.75

So, 2  Tube Lights are needed

Number of Fans =  1.65

So, 2 Fans are needed.

- Living and Dining Room:

Area : 19x16.5 ft^2 = 29.13 m^2

Illuminance , E = 100 Lumen/m^2

Light Loss Factor & Utilization Factor , LLF x UF = 0.7

Number of lights per illuminaire, n= 1

Flux = 1250 Lumen

Number of lights , N = 3.3

So, 3 Lights are needed

Number of Fans =  2.27

So, 2 Fans are needed.

- KITCHEN

Area : 10x9 ft^2= 8.37 m^2

Illuminance , E = 100 Lumen/m^2

Light Loss Factor & Utilization Factor , LLF x UF = 0.7

Number of lights per illuminaire , n= 1

Flux = 1250 Lumen

Number of lights , N = 1.91

So, 2 light Bulbs are needed.

- TOILET

Area : 9x6 ft^2= 5.02 m^2

Illuminance , E = 100 Lumen/m^2

Light Loss Factor & Utilization Factor , LLF x UF = 0.7

Number of lights per illuminaire , n= 1

Flux = 1250 Lumen

Number of lights , N = 0.57

So, 1 light Bulb is needed.

- Generator Room:

Area : 10x9 ft^2= 8.36 m^2

Illuminance , E = 100 Lumen/m^2

Light Loss Factor & Utilization Factor , LLF x UF = 0.7

Number of lights per illuminaire , n= 1

Flux = 1250 Lumen

Number of lights , N = 0.95

So, 1 light Bulb and a tube light is needed.

- Guard Room:

Area : 10x9 ft^2= 8.36 m^2

Illuminance , E = 100 Lumen/m^2

Light Loss Factor & Utilization Factor , LLF x UF = 0.7

Number of lights per illuminaire , n= 1

Flux = 1250 Lumen

Number of lights , N = 0.95

So, 1 light Bulb and a tube light is needed.

Number of Fans =  0.9

So, 1 Fan is needed.

- Garage:

Area : 34.58x29.5 ft^2= 94.76 m^2

Illuminance , E = 100 Lumen/m^2

Light Loss Factor & Utilization Factor , LLF x UF = 0.7

Number of lights per illuminaire , n= 1

Flux = 1250 Lumen

Number of lights , N = 10.83

So, 11 light Bulbs are needed.

- Veranda:

Area : 10x5 ft^2= 4.64 m^2

Illuminance , E = 100 Lumen/m^2

Light Loss Factor & Utilization Factor , LLF x UF = 0.7

Number of lights per illuminaire , n= 1

Flux = 1250 Lumen

Number of lights , N = 0.37

So, 1 light Bulb is needed.

- CALCULATION FOR CONDUITS :

- Formulae :

for ampere rating , I = PV*pf (A)

Power Factor , pf = 0.7

- Apparatus :

Energy Saving Bulb, K = 20 W

Tube Light, L = 20 W

Ceiling Light , L  = 20 W

Ceiling Fan ,F = 100 W

Switch-Board Socket , SS= 100 W

Exhaust Fan ,EF = 60 W

- CALCULATIONS FOR

SDB SB

- CKT-1 RATING :

It consists of SB3 & SB6:

P1 = 20*3+60*2+100 = 280W

I = 280220*0.7 = 1.81A

According to the chart, this current rating is below 5A.

So, 2 x 1.5 rm BYM + 1.5 rm BYA ECC is used.

- CKT-2 RATING :

It consists of SB1 & SB4:

P2 = 20*6+100*4+100*2= 720 W

I = 720220*0.7 = 4.67 A

According to the chart , this current rating is below 5A .

So, 2 x 1.5 rm BYM + 1.5 rm BYA ECC are used.

- CKT-3 RATING :

It consists of SB2:

P3 = 20*2+100*2+100= 340 W

I = 340220*0.7 = 2.2 A

According to the chart, this current rating is below 5A.

So, 2 x 1.5 rm BYM + 1.5 rm BYA ECC is used.

- CKT-4 RATING :

It consist of SB5:

P4 =20*2+100*2+100 = 340 W

I = 340220*0.7 = 2.2 A

According to the chart, this current rating is below 5A.

So, 2 x 1.5 rm BYM + 1.5 rm BYA ECC is used.

- CKT-5 RATING :

It consist of SB7:

P5=20W

I = 20220*0.7 = 0.13 A

According to the chart, this current rating is below 5A.

So, 2 x 1.5 rm BYM + 1.5 rm BYA ECC is used.

SWITCH BOARD (SB) :   From switch board to all fittings we will use  2 x 1.5 rm B

- CALCULATIONS FOR SDB(Ground) SB(Ground)

SUB DISTRIBUTION BOARD (SDB)

- CKT-1 RATING :

It consists of SB1g,SB2g & SB3g:

P1 = 20*5+60*4+100*2 = 540W

I = 540220*0.7 = 3.5A

According to the chart, this current rating is below 5A.

So, 2 x 1.5 rm BYM + 1.5 rm BYA ECC is used.

- CKT-2 RATING :

It consists of SB4g:

P2 = 20*11= 220 W

I = 220220*0.7 = 1.42A

According to the chart , this current rating is below 5A .

So, 2 x 1.5 rm BYM + 1.5 rm BYA ECC are used

- CALCULATIONS FOR ESDB  ESB

EMERGENCY SUB DISTRIBUTION BOARD (SDB)

- CKT-1 RATING :

It consists of ESB3 &ESB6

P1 =20*2 = 40 W

I = 40220*0.7 = 0.26 A

According to the chart, this current rating is below 5A.

So, 2 x 1.5 rm BYM + 1.5 rm BYA ECC is used.

- CKT-2 RATING :

It consists of ESB1 & ESB4:

P2 = 20*2+100*2 =240 W

I = 240220*0.7 = 1.56 A

According to the chart, this current rating is below 5A.

So, 2 x 1.5 rm BYM + 1.5 rm BYA ECC is used.

- CKT-3 RATING :

It consists of ESB2

P3 = 20+100 = 120 W

I = 120220*0.7 = 0.78 A

According to the chart, this current rating is below 5A.

So, 2 x 1.5 rm BYM + 1.5 rm BYA ECC is used.

- CKT-4 RATING :

It consists of ESB5

P4 = 100+20 = 120 W

I = 120220*0.7 = 0.78 A

According to the chart, this current rating is below 5A.

So, 2 x 1.5 rm BYM + 1.5 rm BYA ECC is used.

- CALCULATIONS FOR ESDB(Ground) ESB(Ground)

EMERGENCY SUB DISTRIBUTION BOARD (SDB)

- CKT-1 RATING :

It consists of ESB1g  & ESB3g:

P1 = 20*2 = 40W

I = 40220*0.7 = 0.26A

According to the chart, this current rating is below 5A.

So, 2 x 1.5 rm BYM + 1.5 rm BYA ECC is used.

- CKT-2 RATING :

It consists of ESB4g:

P2 = 20 W

I = 20220*0.7 = 0.13A

According to the chart , this current rating is below 5A .

So, 2 x 1.5 rm BYM + 1.5 rm BYA ECC are used.

- CALCULATIONS FOR MDB  SDB

SDB Load = Total load x 0.7 + Total P socket load x 0.2 + Total Q socket load x 0.3

SDB Current  = SDB LoadV*pf  A

Total Load = P1 + P2 + P3 + P4 +P5= 280+720+340+340+20  = 1700 W

We did not use ant P-socket.

- Q - SOCKET:

Current Rating : 20 A

Power factor : 0.3

Total Q socket used : 3

Power for Q socket : 4000W

- CIRCUIT BREAKER :

20A SP MCB is needed from MDB  Q Socket

- CONDUIT :

2 x 6 rm BYM + 6 rm BYA ECC are used.

SDB Load = 1700 *0.7+ (3* 4000 *0.3) = 4790 W

SDB Current  = 4790220*0.7  A = 31.1A

- CIRCUIT BREAKER :

40A SP MCCB is needed from MDB to SDB

- CONDUIT :

2 x 16 rm BYM + 16 rm BYA ECC are used.

- CALCULATIONS FOR EMDB  ESDB

ESDB Load = Total load x 0.7 + Total P socket load x 0.2 + Total Q socket load x 0.3

ESDB Current  = ESDB LoadV*pf  A

Total Load = P1 + P2 + P3 + P4= 40+240+120+120 = 520 W

No P/Q sockets were used in the emergency circuits

ESDB Load = 520*0.7=364 W

ESDB Current  = 364220*0.7  A = 2.36 A

- CIRCUIT BREAKER :

5A SP MCCB is needed from EMDB to ESDB

- CONDUIT :

2 x 1.5 rm BYM + 1.5 rm BYA ECC are used.

- CALCULATIONS FOR MDB  SDB(G)

SDB Load = Total load x 0.7 + Total P socket load x 0.2 + Total Q socket load x 0.3

SDB Current  = SDB LoadV*pf  A

Total Load = P1 + P2 = 540 + 220= 760W

We did not use ant P/Q-socket.

SDB(G) Load = 760*0.7=532W

SDB(G) Current  = 532220*0.7  A = 3.45A

- CIRCUIT BREAKER :

5A SP MCCB is needed from MDB to SDB

- CONDUIT :

2 x 1.5 rm BYM + 1.5 rm BYA ECC are used.

- CALCULATIONS FOR EMDB  ESDB(G)

ESDB Load = Total load x 0.7 + Total P socket load x 0.2 + Total Q socket load x 0.3

ESDB Current  = ESDB LoadV*pf  A

Total Load = P1 + P2 = 40+20 = 60 W

No P/Q sockets were used in the emergency circuits

ESDB(G) Load = 60*0.7=42 W

ESDB(G) Current  = 42220*0.7  A = 0.27 A

- CIRCUIT BREAKER :

5A SP MCCB is needed from EMDB to ESDB

- CONDUIT :

2 x 1.5 rm BYM + 1.5 rm BYA ECC are used.

- CALCULATIONS FOR EMDB:

EMDB Load = Total ESDB Load + Lift load(Already multiplied with 0.7 so no need to multiply again)

Total ESDB Load = 5x ESDB Load+ ESDB(G) Load

EMDB Current =  EMDB Load3*line voltage*pf

Phase Voltage = 220V

Line Voltage = 381.05 V

Pf = 0.7

No lift in our design

ESDB Load  = 364 W, ESDB(G) Load=42W

Total ESDB Load = 5*364+42=1862 W

EMDB Load=1862W

EMDB Current = 18623*381.05*0.7 = 4.03 A

10 A TP MCCB is needed for EMDB to MDB.(Just to be safe)

- CALCULATIONS FOR MDB:

MDB Load = Total SDB Load + (EMDB Load + Pump)

Total SDB Load = 5 x SDB Load + SDB(G) Load

MDB Current =  MDB Load3*line voltage*pf

Phase Voltage = 220V

Line Voltage = 381.05 V

Pf = 0.95(due to PFI plant)

SDB Load  = 4790 W

SDB(G) Load=532W

Pump load=5000W

Total SDB load=5*4790+532=24482W

MDB Load = 24482+5000 = 29482W

MDB Current = 294823*381.05*0.95 = 47.02 A

50 A TP MCCB is needed for MDB to Main line.

- CALCULATIONS FOR TRANSFORMER:

S = 3 VI = 3 * 220 * 47.02 =31.033kVA

So, 11/0.415 KVA, 50Hz, 50 KVA ,DYN 11,OIL IMMERSED TRANSFORMER WITH 4-6% LINE IMPEDANCE IS REQUIRED.

- CALCULATIONS AIR TERMINALS:

Total circumference = 2 *(61.1+40.7) = 203.6 feet=62 m (approx.)

Air terminals should be placed  at 20m distance.

Air terminal number = 62/20 = 3.1 =3(After rounding up)

- CALCULATIONS FOR MINIMAL  LOAD DENSITY:

According to RAJUK,  for Air conditioned dwelling abodes 100 W/m^2 should be unit load.

In our case minimal Load = Total Load/Apartment size in m^2

= (4790+532)/(40.7*27.8)*0.3048*0.3048

= 50.62 W/m^2

- Calculations for PFI Plant:

cosθ = 0.7 sin θ= (1-cosθ)2 0.714

3Vlsinθ= Ptanθ = 30.1 KVAR

After Pf improvement, sinθ = 1

sino

=36.88A

So, 50 A TP MCCB is needed from PFI to MDB